# Tinkercad Wiring Guide for Fire-Fighting Robot

This guide provides step-by-step instructions for wiring the fire-fighting robot circuit in Tinkercad, based on the provided diagram and the generated Tinkercad-style image. Refer to the generated image (tinkercad_circuit.png) for a visual representation of these connections.

## 1. Components Required

Ensure you have the following components in your Tinkercad workspace:

- Arduino Uno

- L298N Motor Driver

- DC Motors (x4)

- Flame Sensors (x3)

- HC-SR04 Ultrasonic Sensor

- 1-Channel Relay Module

- Water Pump

- 12V Battery Pack (e.g., 8xAA batteries in series for 12V)

- 3.7V Li-ion Battery (e.g., 18650 battery)

- Buck Converter (12V to 5V)

- Breadboard (full-sized)

- Jumper Wires (various colors)

## 2. Power Distribution

- 12V Battery Pack to L298N and Buck Converter:

  - Connect the positive (+) terminal of the 12V Battery Pack to the +12V terminal of the L298N Motor Driver.

  - Connect the negative (-) terminal of the 12V Battery Pack to the GND terminal of the L298N Motor Driver.

  - Connect the positive (+) terminal of the 12V Battery Pack to the IN+ terminal of the Buck Converter.

  - Connect the negative (-) terminal of the 12V Battery Pack to the IN- terminal of the Buck Converter.

- Buck Converter to Breadboard 5V Rail:

  - Connect the OUT+ terminal of the Buck Converter to the positive (+) rail of the breadboard.

  - Connect the OUT- terminal of the Buck Converter to the negative (-) rail (GND) of the breadboard.

- Arduino Power:

  - Connect the 5V pin on the Arduino to the positive (+) rail of the breadboard.

  - Connect one of the GND pins on the Arduino to the negative (-) rail of the breadboard.

- 3.7V Li-ion Battery to Water Pump via Relay:

  - Connect the positive (+) terminal of the 3.7V Li-ion Battery to the COM (Common) terminal of the Relay Module.

  - Connect the NO (Normally Open) terminal of the Relay Module to the positive (+) terminal of the Water Pump.

  - Connect the negative (-) terminal of the Water Pump to the negative (-) terminal of the 3.7V Li-ion Battery.

## 3. Arduino Connections

### Flame Sensors (x3)

- Flame Sensor 1 (Left):

  - VCC to Breadboard + rail

  - GND to Breadboard - rail

  - DO (Digital Output) to Arduino Analog Pin A0

- Flame Sensor 2 (Front):

  - VCC to Breadboard + rail

  - GND to Breadboard - rail

  - DO to Arduino Analog Pin A1

- Flame Sensor 3 (Right):

  - VCC to Breadboard + rail

  - GND to Breadboard - rail

  - DO to Arduino Analog Pin A2

### HC-SR04 Ultrasonic Sensor

- VCC to Breadboard + rail

- GND to Breadboard - rail

- Trig to Arduino Digital Pin 3

- Echo to Arduino Digital Pin 2

### L298N Motor Driver

- ENA to Arduino Digital Pin 9

- IN1 to Arduino Digital Pin 8

- IN2 to Arduino Digital Pin 7

- IN3 to Arduino Digital Pin 6

- IN4 to Arduino Digital Pin 5

- ENB to Arduino Digital Pin 10

### 1-Channel Relay Module

- VCC to Breadboard + rail

- GND to Breadboard - rail

- IN to Arduino Digital Pin 12

## 4. Motor Connections

Connect the four DC Motors to the L298N Motor Driver outputs:

- Motor 1 (Front Left): Connect its two terminals to OUT1 and OUT2 of the L298N.

- Motor 2 (Front Right): Connect its two terminals to OUT3 and OUT4 of the L298N.

- Motor 3 (Rear Left): Connect its two terminals to OUT1 and OUT2 of the L298N (or use separate outputs if available, but the diagram implies sharing for simplicity).

- Motor 4 (Rear Right): Connect its two terminals to OUT3 and OUT4 of the L298N (or use separate outputs if available).

Note: The L298N typically controls two motors per side. For four motors, you would usually connect two motors in parallel to each output pair (e.g., Motor 1 and Motor 3 to OUT1/OUT2, and Motor 2 and Motor 4 to OUT3/OUT4). Ensure the motors are wired correctly for forward/backward movement.