# Arduino Flame Sensor Direction Test

## Project Goal

This project tests three flame sensors connected to an Arduino UNO. The system reads flame intensity from LEFT, CENTER, and RIGHT sensors and determines the fire direction.

## Sensor Connections

| Sensor | Arduino Pin |
| --- | --- |
| Left Flame Sensor | A0 |
| Center Flame Sensor | A1 |
| Right Flame Sensor | A2 |
| Relay Placeholder | D7 |

## Threshold Explanation

The flameThreshold value determines when the system decides that a flame exists.

Typical analog readings:
- No flame: 700 to 1023
- Flame detected: 0 to 500

Threshold tuning guide:

300:
- Very strict detection
- Detects only strong or close flames
- Reduces false alarms
- Best for bright environments

500:
- Recommended starting value
- Balanced sensitivity
- Good for normal indoor testing
- Detects medium-distance flames reliably

700:
- Very sensitive detection
- Detects weak or distant flames
- May create false alarms from room lighting or sunlight
- Useful for early testing and calibration

## Direction Detection Logic

The Arduino compares all three sensor values. The sensor with the LOWEST reading is considered closest to the flame.

## Example

LEFT = 800
CENTER = 320
RIGHT = 760

Result: Fire detected CENTER

## Circuit Diagram

## Arduino Source Code

/*
  ============================================
   3 Flame Sensor Direction Test
  ============================================

  Sensors:
    Left   -> A0
    Center -> A1
    Right  -> A2

  Goal:
    - Read flame sensor values
    - Print values every 500ms
    - Detect fire direction:
        LEFT / CENTER / RIGHT
    - Print:
        "No fire detected"
        "Fire detected LEFT"
        "Fire detected CENTER"
        "Fire detected RIGHT"

  Future placeholder:
    relay_pin = 7   // pump relay (not used yet)
*/

const int leftSensor   = A0;
const int centerSensor = A1;
const int rightSensor  = A2;

// Future relay for water pump
const int relay_pin = 7;   // NOT USED YET

// Flame detection threshold
const int flameThreshold = 500;

void setup()
{
  Serial.begin(9600);

  pinMode(leftSensor, INPUT);
  pinMode(centerSensor, INPUT);
  pinMode(rightSensor, INPUT);

  // Relay placeholder
  pinMode(relay_pin, OUTPUT);
  digitalWrite(relay_pin, LOW);

  Serial.println("=== Flame Sensor Test Started ===");
}

void loop()
{
  // Read sensor values
  int leftValue   = analogRead(leftSensor);
  int centerValue = analogRead(centerSensor);
  int rightValue  = analogRead(rightSensor);

  // Print sensor values
  Serial.print("LEFT: ");
  Serial.print(leftValue);

  Serial.print(" | CENTER: ");
  Serial.print(centerValue);

  Serial.print(" | RIGHT: ");
  Serial.println(rightValue);

  // Determine lowest value
  // Lower value = stronger flame signal
  int minValue = leftValue;
  String direction = "LEFT";

  if (centerValue < minValue)
  {
    minValue = centerValue;
    direction = "CENTER";
  }

  if (rightValue < minValue)
  {
    minValue = rightValue;
    direction = "RIGHT";
  }

  // Check flame threshold
  if (minValue < flameThreshold)
  {
    Serial.print("Fire detected ");
    Serial.println(direction);
  }
  else
  {
    Serial.println("No fire detected");
  }

  Serial.println("--------------------------------");

  delay(500);
}