# Flame Sensor Direction Detection System (Upgraded)

# Overview

This project uses 3 flame sensors (Left, Center, Right) to detect fire direction and determine the strongest flame source using analog readings.

# Sensors Used

Left Sensor -> A0
Center Sensor -> A1
Right Sensor -> A2

# Key Improvements

- Flame threshold filtering (flameThreshold = 500)
- Directional difference filtering (minDifference = 30)
- Reduced false alarms
- More stable flame direction detection

# How It Works

1. The system reads analog values from 3 flame sensors.
2. Lower value means stronger flame detection.
3. The system finds the minimum value (strongest flame).
4. It compares the difference between the strongest and second strongest sensor.
5. If the flame is strong enough AND clearly dominant, direction is confirmed.

# Threshold Logic

flameThreshold = 500 → minimum required flame intensity
minDifference = 30 → required difference between sensors for stable direction

# Benefits

- Reduces false flame detection
- Prevents unstable direction switching
- Improves accuracy in real environments
- Better performance in noisy sensor conditions