# 4-Wheel Arduino Robot Project

## Project Description

This project tests a 4-wheel DC robot using an Arduino and L298N motor driver. The robot performs forward, backward, left, and right movements, then follows a square path using delay-based timing without sensors or PID control.

## Components Used

Arduino UNO or Nano

L298N Motor Driver

4 DC Motors

Robot Chassis

Battery Pack

Jumper Wires

Optional Relay Module (future water pump control)

## Pin Mapping

| L298N Pin | Arduino Pin | Purpose |
| --- | --- | --- |
| IN1 | 8 | Left motor direction |
| IN2 | 9 | Left motor direction |
| IN3 | 10 | Right motor direction |
| IN4 | 11 | Right motor direction |
| ENA | 5 | Left motor speed PWM |
| ENB | 6 | Right motor speed PWM |
| Relay | 7 | Reserved water pump relay |

## Circuit Diagram

## Arduino Code

/*
4-Wheel Robot Test using Arduino + L298N
*/

const int IN1 = 8;
const int IN2 = 9;
const int IN3 = 10;
const int IN4 = 11;

const int ENA = 5;
const int ENB = 6;

const int relay_pin = 7; // Reserved for water pump

int motorSpeed = 200;

void setup() {
  pinMode(IN1, OUTPUT);
  pinMode(IN2, OUTPUT);
  pinMode(IN3, OUTPUT);
  pinMode(IN4, OUTPUT);

  pinMode(ENA, OUTPUT);
  pinMode(ENB, OUTPUT);

  pinMode(relay_pin, OUTPUT);
  digitalWrite(relay_pin, LOW);

  analogWrite(ENA, motorSpeed);
  analogWrite(ENB, motorSpeed);

  delay(2000);

  moveForward();
  delay(3000);
  stopMotors();
  delay(1000);

  moveBackward();
  delay(3000);
  stopMotors();
  delay(1000);

  turnLeft();
  delay(3000);
  stopMotors();
  delay(1000);

  turnRight();
  delay(3000);
  stopMotors();
  delay(2000);
}

void loop() {
  for (int i = 0; i < 4; i++) {
    moveForward();
    delay(3000);

    stopMotors();
    delay(500);

    turnLeft();
    delay(700);

    stopMotors();
    delay(500);
  }

  stopMotors();

  while (true);
}

void moveForward() {
  digitalWrite(IN1, HIGH);
  digitalWrite(IN2, LOW);
  digitalWrite(IN3, HIGH);
  digitalWrite(IN4, LOW);
}

void moveBackward() {
  digitalWrite(IN1, LOW);
  digitalWrite(IN2, HIGH);
  digitalWrite(IN3, LOW);
  digitalWrite(IN4, HIGH);
}

void turnLeft() {
  digitalWrite(IN1, LOW);
  digitalWrite(IN2, HIGH);
  digitalWrite(IN3, HIGH);
  digitalWrite(IN4, LOW);
}

void turnRight() {
  digitalWrite(IN1, HIGH);
  digitalWrite(IN2, LOW);
  digitalWrite(IN3, LOW);
  digitalWrite(IN4, HIGH);
}

void stopMotors() {
  digitalWrite(IN1, LOW);
  digitalWrite(IN2, LOW);
  digitalWrite(IN3, LOW);
  digitalWrite(IN4, LOW);
}

## Movement Logic

Forward: Both left and right motors rotate forward.
Backward: Both left and right motors rotate backward.
Left Turn: Left motors backward and right motors forward.
Right Turn: Left motors forward and right motors backward.
Square Path: Move forward for 3 seconds and turn left for a timed 90-degree rotation.